Adding the MCS7720 drivers to the Platform

Introduction:

This document provides the information about how to add/include  MCS7720  device drivers to a windows CE .NET 5.0 platform.

The following steps explains how to add/include the   MCS7720 device drivers to Windows CE .NET 5.0 platform.

- Create and Build a CEPC platform

- After building the platform add the following entries in the platform.reg file

[HKEY_LOCAL_MACHINE\Drivers\USB\LoadClients\38672_30496\255_0_255\Default\Mos20UsbSerialClass]

"Dll"="USer20Ce.DLL"

[HKEY_LOCAL_MACHINE\Drivers\USB\ClientDrivers\Mos20UsbSerialClass]

"Dll"="USer20Ce.DLL"

- Add the following entries in the platform.bib file

USer20Ce.dll  $(_FLATRELEASEDIR)\ USer20Ce.dll               NK

- Copy the USer20Ce.dll in to the following folder.

Drive:>\WINCE500\PUBLIC\CEPlatform\RelDir\CEPC_X86Release

- Then build the platform to effect the above changes.

This platform image contains the  MCS7720 device driver and you can connect  MCS7720 to the target machine and configure after downloading the platform OS image.